STEREO IMPACT FM2 SWEA Actel Replacement Instructions

The FM2 SWEA Actel (on the DAC board) was determined to be faulty by diagnostic test.  The Actel needs to be replaced.

- Remove the Actel on the SWEA FM2 DAC board.  Keep the part for failure analysis.

- COMPLETED BY ________________________	DATE _________________________________

- Install a new Actel at the same location.  Use an RTSX32SU-1CQ208BX109 part provided by GSFC (5962-0150806QYC), programmed with the “FlightssfRevF2_Designer7.AFM’ file at GSFC

- Lot Date Code __________, Serial number ___________

- COMPLETED BY ________________________	DATE _________________________________

- INSPECTED  BY ________________________	DATE _________________________________

- Stake the part with Uralane 5750

- Mix Info______________________

- Cure Info_____________________

- Conformal Coat with Uralane 5753.

- Mix Info______________________

- Cure Info_____________________

- COMPLETED BY ________________________	DATE _________________________________

Approved by: _______________________________	Date: ____________________