STEREO IMPACT SWEA FM2 LVPS REPAIR INSTRUCTIONS

- The T2 transformer on the FM2 SWEA LVPS seems to be broken.  There is no signal on the secondary side, probably indicating a break on the primary winding.   Impendence measurements also seem to confirm this.   Also the bobbin seems to be floating in the core, indicating the epoxy that is supposed to hold it in was either omitted or has broken.

# Replace FM2 SWEA LVPS T2 transformer

- Remove the existing T2 transformer from the board, taking care not to damage it so we can do a failure analysis on the part.

- Install a replacement T2 transformer (part STEREO 027)

- Part serial number_____________________________

- Return the board to David Curtis for test

- Stake the part to the board using Uralane 5753.  Record material date code, mix information, and cure times in FM2 SWEA LVPS traveler.

- Coat the part to the board using Uralane 5750.  Record material date code, mix information, and cure times in FM2 SWEA LVPS traveler

- COMPLETED BY ________________________	DATE _________________________________

Approved by: _______________________________	Date: ____________________